## Chapter 3: Reactive Navigation & Obstacle Avoidance

While knowing global kinematics formulas is essential, real-world environments are full of unmapped obstacles, shifting objects, and dynamic targets. A robust robot cannot rely purely on pre-calculated routes; it must react immediately to live sensor streams. This chapter explores local, sensor-driven reactive behaviors designed to keep your robot moving safely without crashing.

### 3.1 Sensor-Driven Reactive Behaviors

In reactive navigation, the robot's actions are tightly coupled with immediate sensor inputs. Instead of building a complex map first, the robot acts on a direct loop: if a sensor detects an object too close, the control law instantly modifies motor velocities to veer away. This architecture provides low-latency responses ideal for avoiding collisions in real time.

### 3.2 Bug Algorithms

One of the earliest formal methods for reactive navigation is the family of Bug algorithms. These algorithms combine simple straight-line goal-seeking with wall-following behaviors:

- Bug 1: The robot drives directly toward the target until it encounters an obstacle. It then circumnavigates the entire perimeter of the obstacle to find the point closest to the goal, returns to that point, and departs toward the goal.

- Bug 2: The robot establishes a virtual direct line from the start position to the goal (the m-line). When it hits an obstacle, it follows the boundary until it intersects the m-line again at a position closer to the goal, then resumes its direct path.

### 3.3 Artificial Potential Fields

A more fluid, continuous approach to reactive steering is the Artificial Potential Field method. Imagine the robot moving through a virtual landscape where forces push and pull it across the floor:

- Attractive Fields: The target goal generates a virtual gravitational pull, drawing the robot toward its coordinates with a force that typically scales with distance.

- Repulsive Fields: Every detected obstacle acts as a magnetic hill, generating a powerful repulsive force that pushes the robot away. The closer the robot gets to the obstacle, the stronger the pushback.

The robot calculates the total virtual force vector by adding these forces together:

Ftotal=Fattractive+Frepulsive

The robot then maps this combined force vector directly to its wheel velocities to determine its steering path.

#### The Local Minima Trap

A major limitation of potential fields is the local minima trap. If a robot encounters a U-shaped barrier or stands directly between balanced obstacles, the attractive force pulling it forward can exactly equal the repulsive forces pushing it back:

Ftotal=0

When these forces cancel out before reaching the goal, the robot stalls or gets stuck permanently because its algorithmic landscape has no slope to follow.

### 3.4 ActiveCode Exercise: Reactive Wall Avoidance State Machine

Complete the state-machine loop below. This script simulates a distance sensor array (left_dist, front_dist, right_dist) on a mobile robot. Fill in the conditional logic blocks to guide the robot away from walls.

Python

def reactive_steering(left_dist, front_dist, right_dist):

"""

Returns a steering action string: "FORWARD", "STEER_LEFT", or "STEER_RIGHT"

Clear path threshold is 1.0 meter. Danger zone is under 0.5 meters.

"""

# TODO: Write logic to handle a wall blocking the front path

if front_dist < 0.5:

# If blocked in front, turn toward the side with more open space

if left_dist > right_dist:

return "STEER_LEFT"

else:

return "STEER_RIGHT"

# TODO: Write logic if an obstacle approaches too close on the left side

elif left_dist < 0.5:

return "STEER_RIGHT"

# TODO: Write logic if an obstacle approaches too close on the right side

elif right_dist < 0.5:

return "STEER_LEFT"

else:

return "FORWARD"

# Test Case: Wall detected closely ahead and slightly to the right side

action = reactive_steering(left_dist=1.2, front_dist=0.4, right_dist=0.3)

print(f"Sensor Readings -> Front: 0.4m, Left: 1.2m, Right: 0.3m")

print(f"Calculated Reactive Action: {action}")

### 3.5 Conceptual Check

Question: A differential drive robot navigating via potential fields drives straight into a symmetric, U-shaped cubicle. The goal position lies directly behind the back wall of the cubicle. The robot drives into the center of the pocket, stops, and stands completely still without reaching the target. What is the structural root cause of this failure?

- (A) The robot experienced a catastrophic kinematics inversion error.

- (B) The robot hit a local minimum where the attractive force toward the goal was perfectly countered by the combined repulsive forces of the walls.

- (C) The ultrasonic distance sensor array suffered an open-loop feedback drift.

- (D) The Bug 2 algorithm m-line configuration parameters were set to zero.

Check Answer: The correct answer is (B). This is a classic local minimum issue. Because the layout is perfectly symmetric, the vector sum of the repulsive forces from the three walls points directly opposite the goal's attractive force, stranding the robot in a local equilibrium point.