## Chapter 5: Spatial Mapping & Localization Basics

In previous chapters, we provided our robot with static maps or reactive sensory behaviors to avoid sudden obstacles. However, in a real-world deployment—such as tracking objects or building an arena map during your lab tasks—a robot must answer two fundamental questions simultaneously: “What does the world look like?” and “Where am I within it?”. This chapter explores the basics of how robots build representations of their space and track their coordinates over time.

### 5.1 The Paradox of SLAM

Simultaneous Localization and Mapping (SLAM) is a foundational algorithmic challenge in robotics. It represents a classic "chicken-and-egg" dilemma:

- To estimate its own position (localization), a robot needs a highly accurate map of its environment.

- To construct an accurate map (mapping), the robot must know exactly where it is located as it records sensor measurements.

[ Mapping ]

│  (Needs accurate robot position)

▼

┌───────────────────────┐

│   The SLAM Loop       │

└───────────────────────┘

▲

│  (Needs accurate environmental map)

[ Localization ]

When a robot is dropped into an entirely unfamiliar arena, it must solve both problems concurrently. It updates its estimated position while incrementally layering sensor measurements into a shared spatial model.

### 5.2 Dead Reckoning vs. Sensor Correction

The most primitive way a mobile robot estimates its current position is via dead reckoning (often utilizing odometry).

#### Odometry & Odometer Drift

Odometry relies on hardware sensors like wheel encoders to measure how many rotations each wheel has made. By tracking these turns over time, the robot uses the kinematic formulas we studied in Chapter 2 to update its relative coordinates (x,y,θ).

However, dead reckoning suffers from a fatal flaw: odometer drift. Because odometry calculates position by continually adding new measurements to old estimations, any microscopic error is compounded over time. Small real-world physical discrepancies—such as wheel slippage on smooth floors, uneven tire wear, or minor floor bumps—introduce systemic and non-systemic noise. These errors stack up continuously, causing the robot’s estimated location to drift away from its actual physical position.

#### Sensor Correction & Landmark Maps

To fix this accumulating drift, robots rely on sensor correction via external observations. Instead of trusting internal odometry blindly, the robot looks outward using perception devices (like distance sensors or an AI vision sensor) to identify static features called landmarks.

A landmark-based map stores the precise coordinates of highly recognizable environmental features (such as fixed arena posts, wall corners, or distinct static targets). Every time the robot recognizes a known landmark, it measures its distance and angle relative to that feature. It then uses this data to correct its internal coordinate estimates, resetting the accumulated drift error back down toward zero.

### 5.3 ActiveCode Exercise: Simulating Odometer Drift

Run the simulation below to see how random Gaussian noise affects dead reckoning. This script updates a robot's estimated position over 50 discrete time steps. We introduce a tiny variance in the wheel encoder data at each step to mimic real-world wheel slippage.

Notice how the Estimated Path drifts further away from the Ground Truth as the steps advance.

Python

import random

def simulate_odometry_drift(steps=50, true_step_distance=1.0, noise_std_dev=0.15):

true_x = 0.0

estimated_x = 0.0

print(f"{'Step':<6} | {'True X':<10} | {'Estimated X':<12} | {'Accumulated Drift':<15}")

print("-" * 50)

for step in range(1, steps + 1):

# The robot physically moves forward by the true distance

true_x += true_step_distance

# Encoders misread the movement slightly by injecting Gaussian noise

simulated_noise = random.gauss(0, noise_std_dev)

measured_movement = true_step_distance + simulated_noise

# Dead reckoning estimates the new position using the noisy measurement

estimated_x += measured_movement

drift = estimated_x - true_x

if step % 5 == 0 or step == 1:

print(f"{step:<6} | {true_x:<10.2f} | {estimated_x:<12.2f} | {drift:<15.2f}")

print("-" * 50)

print(f"Final true position: {true_x:.2f} meters")

print(f"Final estimated position: {estimated_x:.2f} meters")

print(f"Total error due to uncorrected drift: {abs(drift):.2f} meters")

# Execute the simulation

simulate_odometry_drift()

### 5.4 Conceptual Check: Fill-in-the-Blank

Instructions: Complete the paragraph below by choosing the correct terms from the following options: odometer drift, sensor correction, landmarks, dead reckoning.

A mobile robot navigating an unknown soccer field relies on internal wheel encoders to estimate its position through a process called __________. While this approach works well over short distances, surface friction changes and minor wheel slips quickly introduce __________, making the position data unreliable over time. To fix this, the robot uses its vision sensor to track distinct, fixed environmental features known as __________. By measuring its relative distance to these features, the robot performs __________ to update its internal coordinates and ensure it does not lose its way.

Check Answer: >

1. dead reckoning

2. odometer drift

3. landmarks

4. sensor correction