## Chapter 9: Introduction to ROS2 Architecture

Up to this point, you have been writing self-contained scripts to control your VEX robot. While a monolithic code layout works fine for simpler tasks, complex autonomous applications—like coordinating vision tracking, trajectory generation, and motor execution during a soccer competition—require multiple tasks to run at the same time. To manage this complexity, modern robotics relies on specialized middleware frameworks. This chapter introduces ROS2 (Robot Operating System 2), the industry-standard software architecture for building distributed robot applications.

### 9.1 Middleware Mechanics & The ROS2 Graph

Despite its name, ROS2 is not a traditional operating system like Windows or Linux. Instead, it is a middleware framework—a software layer that sits on top of a host operating system to provide communication, hardware abstraction, and message management services across diverse software processes.

The backbone of any ROS2 system is the ROS2 Graph. The graph is a network of independent components that process data and communicate with each other concurrently.

[ /camera_node ] ─────── ( /raw_image Topic ) ───────> [ /vision_processor_node ]

│

│ ( /detection_coordinates Topic )

▼

[ /motor_driver_node ] <─── ( /cmd_vel Topic ) <─────── [ /navigation_node ]

### 9.2 The Structural Core: Nodes, Topics, and Messages

To construct this operational graph, the ROS2 framework utilizes three primary abstractions:

#### 1. Nodes

A node is a modular, single-purpose executable process that performs a specific robotic task (e.g., one node reads lidar data, one node calculates an A* path plan, and another drives the wheel motors). Dividing a system into separate nodes improves fault tolerance: if a camera node crashes, the motor driver node can still execute emergency stop commands safely.

#### 2. Topics

Nodes share information asynchronously through a communication channel called a topic. Topics work on a broadcast network model: one node publishes data to a named topic, and any interested node can listen to that topic to receive the stream.

#### 3. Messages (.msg)

Nodes cannot send completely unformatted text or raw memory streams across a topic. Instead, communication is strictly governed by messages (.msg), which are standardized data structures containing explicit, strongly typed fields (e.g., int32, float64, string). This strict mapping ensures that a node written in C++ can seamlessly parse data transmitted by a node written in Python.

#### The Workspace Environment

Before launching any node, you must set up your workspace environment—a dedicated directory structure on your system where your packages are built, compiled, and sourced. Sourcing the workspace tells your operating system exactly where to look for your custom message definitions, node libraries, and executable scripts.

### 9.3 ActiveCode Exercise: Parsing Custom Serialized Messages

Before using live ROS2 communication pipes, you must practice parsing structured data formats. When data travels over a network, it is often serialized into structured strings.

Complete the function below to simulate a ROS2 subscriber parsing an incoming message payload string formatted as a JSON-like stream. Extract the target fields to monitor motor telemetry.

Python

# ActiveCode Exercise: Parsing Simulated Message Payloads

import json

def parse_robot_message(serialized_payload):

"""

Simulates a subscriber callback parsing a telemetry message string.

Expected fields: 'node_src', 'timestamp', 'linear_velocity', 'battery_voltage'

"""

try:

# Deserialize the string payload into a Python dictionary

msg_data = json.loads(serialized_payload)

# TODO: Extract the source node name and linear velocity

source_node = msg_data.get("node_src", "UNKNOWN")

current_vel = msg_data.get("linear_velocity", 0.0)

battery_v   = msg_data.get("battery_voltage", 0.0)

# Diagnostic battery threshold check

if battery_v < 11.0:

status_flag = "WARNING: LOW BATTERY"

else:

status_flag = "BATTERY NOMINAL"

print(f"[{source_node}] Telemetry Parsed Successfully.")

print(f"-> Linear Velocity: {current_vel:.2f} m/s | Status: {status_flag}")

return True

except Exception as e:

print(f"Failed to parse serialized message. Error: {e}")

return False

# Simulated ROS2 Topic transmission payload

mock_topic_msg = '{"node_src": "drivetrain_controller", "timestamp": 171994320, "linear_velocity": 1.45, "battery_voltage": 10.82}'

parse_robot_message(mock_topic_msg)

### 9.4 Conceptual Check: ROS2 Command Line Interface (CLI)

Instructions: Match each ROS2 command line tool with its corresponding architectural function.

| Command Tool | Primary Architectural Target |
| --- | --- |
| 1. ros2 node list | (A) Injects custom manual data into a topic to trigger a reactive movement. |
| 2. ros2 topic echo | (B) Prints out an active directory of every running node process in the graph. |
| 3. ros2 topic pub | (C) Hooks into an active topic stream and prints the live message data to the terminal. |

Select the correct matching combination sequence:

- (A) 1-B, 2-C, 3-A

- (B) 1-A, 2-B, 3-C

- (C) 1-C, 2-A, 3-B

- (D) 1-B, 2-A, 3-C

Check Answer: The correct answer is (A). ros2 node list targets process visualization (B), ros2 topic echo streams live topic data to your screen (C), and ros2 topic pub allows developers to manually publish custom message packages directly into a topic stream (A).