## 1.4 Sensors, Actuators, and the Closed-Loop Robotics Pipeline

Every robot, from a VEX competition robot to a self-driving car, operates by continuously gathering information, making decisions, taking actions, and verifying the results. This process forms a closed-loop system that allows the robot to adapt to changing conditions in its environment.

A typical robotic cycle follows these steps:

- Perception – Gather information from sensors.

- Planning – Decide what action to take.

- Action – Send commands to actuators.

- Verification – Use sensor feedback to determine whether the action was successful.

Sensors → Perception → Planning → Actuators

↑ ↓

└────── Verification / Feedback ───┘

### Perception

During the perception stage, the robot collects data from sensors and converts that data into useful information.

For example:

- A camera detects an object.

- Image classification software identifies the object as a target.

- A distance sensor measures how far away the target is.

At this stage, the robot is answering the question:

"What is happening around me right now?"

### Planning

Once information has been gathered, the robot determines what to do next.

For example:

- A path-planning algorithm calculates the safest route to the target.

- The robot determines how far it must travel.

- The robot decides whether it needs to turn, avoid obstacles, or stop.

At this stage, the robot is answering:

"What should I do to achieve my goal?"

### Action

After the plan is created, commands are sent to actuators.

Examples include:

- Spinning a motor

- Turning a wheel

- Moving a robotic arm

- Opening a gripper

At this stage, the robot is answering:

"How do I physically execute my plan?"

### Verification

After acting, sensors continuously report the robot's actual movement.

If the robot slips, encounters resistance, or turns too far, the sensors detect the difference and report it back to the controller.

The robot is essentially asking:

"Did my action produce the result I expected?"

This feedback closes the loop and allows the robot to make corrections.

## Sensors = Inputs

Sensors provide information about the robot and its environment.

Think of sensors as the robot's eyes, ears, and sense of touch.

Common sensor examples include:

| Sensor | Purpose |
| --- | --- |
| Camera | Detect objects and colors |
| LIDAR | Measure distances and create maps |
| Encoder | Measure wheel or motor rotation |
| GPS | Determine location |
| Inertial Sensor (IMU) | Measure orientation and acceleration |
| Distance Sensor | Detect nearby objects |

### VEX Example

In a VEX robot:

- A Distance Sensor measures how far the robot is from a game object.

- An Optical Sensor detects object colors.

- Motor Encoders measure wheel rotations.

These sensors provide the information needed for autonomous navigation and precise movement.

## Actuators = Outputs

Actuators are devices that convert electrical energy into physical motion.

Think of actuators as the robot's muscles.

Common actuator examples include:

| Actuator | Purpose |
| --- | --- |
| DC Motor | Rotate wheels |
| Servo Motor | Move to precise angles |
| Pneumatic Cylinder | Push or pull mechanisms |
| Gripper | Grab objects |
| Robotic Arm | Lift or manipulate items |

### VEX Example

A VEX robot often uses:

- Smart Motors to drive wheels.

- Intake motors to collect game objects.

- Arm motors to lift mechanisms.

- Claw systems to grab or release objects.

When the robot decides to move, these actuators physically carry out the command.

## Real-World Example 1: Self-Driving Car

A self-driving car uses the same sensor-actuator cycle as a VEX robot, but on a much larger scale.

### Perception

The vehicle gathers information using:

- Cameras

- Radar

- LiDAR

- GPS

The system identifies:

- Cars

- Pedestrians

- Traffic signs

- Lane markings

### Planning

The vehicle calculates:

- Where it should drive

- How fast it should travel

- Whether it needs to stop or change lanes

### Action

The control system sends commands to:

- Steering actuators

- Braking systems

- Throttle controls

### Verification

Wheel encoders and other sensors verify:

- Vehicle speed

- Steering angle

- Position on the road

If the car drifts slightly, the system immediately makes corrections.

### Connection to VEX

This is similar to a VEX robot:

- Seeing a game object.

- Planning a route.

- Driving toward the object.

- Using encoders to verify it moved the correct distance.

The primary difference is complexity and scale.

## Real-World Example 2: Amazon Warehouse Robot

Modern warehouses use autonomous robots to move inventory shelves.

### Perception

Robots use:

- Cameras

- QR code markers

- Distance sensors

to determine their current location.

### Planning

The robot calculates:

- Which shelf to retrieve

- The fastest route to the destination

- How to avoid collisions with other robots

### Action

The robot:

- Drives to the shelf.

- Lifts it.

- Carries it to a packing station.

### Verification

Encoders continuously measure wheel rotations.

If wheel slip occurs or another robot blocks the path, the controller detects the discrepancy and recalculates the route.

### Connection to VEX

Imagine a VEX robot competing in an autonomous challenge:

- Distance Sensor identifies a scoring object.

- Navigation logic plans a path.

- Drive motors move the robot.

- Encoders confirm the robot traveled correctly.

The same principles used in an Amazon warehouse robot are present in a VEX competition robot.

## VEX Case Study: Collecting a Game Object

Suppose a VEX robot must collect a red game object located across the field.

### Perception

- Optical Sensor detects a red object.

- Distance Sensor measures the object's location.

### Planning

- The robot calculates a path.

- The controller determines how far it must drive.

### Action

- Drive motors move the robot forward.

- Intake motors activate.

### Verification

- Encoders measure wheel rotation.

- The robot compares expected distance to actual distance.

- PID control adjusts motor power if the robot drifts or slips.

This entire process may occur dozens of times per second, allowing the robot to react smoothly and accurately to its environment.

### Multiple-Choice Question: Sensors, Actuators, and the Closed-Loop Feedback Loop

A VEX robot is programmed to drive toward a red game object, pick it up, and return to a scoring zone. Which sequence correctly represents the robot's closed-loop operation?

A. Drive motors move → Optical Sensor detects object → Encoders verify movement → Path is planned

B. Optical Sensor detects object → Controller plans a path → Drive motors and intake motors activate → Encoders verify movement and provide feedback ✅

C. Encoders verify movement → Intake motor activates → Optical Sensor detects object → Controller plans a path

D. Controller plans a path → Encoders verify movement → Optical Sensor detects object → Motors activate

Correct Answer: B

Feedback:

- A: Incorrect. The robot must first sense its environment before acting.

- B: Correct. The robot first gathers information, then plans, acts, and finally verifies the results using sensor feedback.

- C: Incorrect. Verification occurs after the robot has taken action.

- D: Incorrect. The robot must gather sensor data before planning.

Explanation:
A closed-loop robotic system follows the cycle:

Sense → Plan → Act → Verify

For a VEX robot:

- Sense: Optical and Distance Sensors detect the game object.

- Plan: The controller calculates how to reach it.

- Act: Motors move the robot and operate the intake.

- Verify: Encoders measure wheel rotation and provide feedback to ensure the robot is moving accurately.

### Multiple-Choice Question: Sensors vs. Actuators

Which of the following correctly pairs a VEX sensor and a VEX actuator?

A. Encoder (Sensor) and Smart Motor (Actuator) ✅

B. Smart Motor (Sensor) and Optical Sensor (Actuator)

C. Battery (Sensor) and Encoder (Actuator)

D. Brain Screen (Sensor) and Distance Sensor (Actuator)

Correct Answer: A

Explanation:
Sensors provide input to the robot, while actuators provide output. An encoder collects information about movement, while a Smart Motor converts electrical energy into physical motion.

### Multiple-Choice Question: Closing the Loop

Why does a VEX robot use encoders during autonomous movement?

A. To determine the color of game objects.

B. To increase motor speed regardless of conditions.

C. To measure wheel rotations and provide feedback so the controller can correct errors. ✅

D. To store the robot's programming code.

Correct Answer: C

Feedback:

- A: Incorrect. Optical Sensors are commonly used for color detection.

- B: Incorrect. Encoders measure movement; they do not directly control speed.

- C: Correct. Encoders provide feedback that helps the robot determine whether it has traveled the expected distance.

- D: Incorrect. Program code is stored in the VEX Brain.

Explanation:
Encoders help "close the loop" by reporting actual wheel movement back to the controller. If the robot slips or drifts, the controller can make corrections using the encoder feedback.