## 3.1 Sensor-Driven Reactive Behaviors

One of the simplest and most effective ways for a robot to interact with its environment is through reactive behavior. In reactive navigation, the robot makes decisions directly from sensor inputs without first building a detailed map of its surroundings. When new information is detected, the robot immediately adjusts its actions. This creates a fast feedback cycle that allows the robot to respond to changing environments in real time.

Reactive behaviors are commonly used when a robot must avoid collisions, follow walls, track objects, or stop before hitting an obstacle. Because the robot does not perform extensive calculations before acting, reactive systems typically have very low latency and can react quickly to sudden changes.

A basic reactive control loop looks like this:

Sensor Input → Decision Rule → Motor Command → Movement

↑ ↓

└──────── Updated Sensor Data ────┘

For example:

IF obstacle distance < 10 cm

THEN turn right

The robot continuously evaluates this rule while driving.

## VEX AIM Robot Example: Obstacle Avoidance

Suppose a VEX AIM robot is driving through a classroom.

### Sensor Input

The AIM Distance Sensor detects a desk 8 cm in front of the robot.

Distance = 8 cm

### Decision Rule

The control program contains:

IF distance < 10 cm

THEN turn right

### Action

The robot slows its left wheel and increases its right wheel speed, causing the robot to turn away from the obstacle.

### Result

The robot avoids the desk without needing a map of the room.

This entire process may occur several times per second.

## VEX AIM Robot Example: Object Following

Reactive behaviors can also be used to follow a target.

### Sensor Input

The Distance Sensor detects a game piece located 50 cm away.

### Decision Rule

IF object is far away

THEN move forward

IF object is too close

THEN stop

### Action

The AIM robot continuously adjusts its speed while approaching the object.

The robot reacts directly to sensor measurements rather than calculating a complete route.

## Real-World Example 1: Robotic Vacuum Cleaners

Most robotic vacuum cleaners use reactive behaviors.

### Sensor Input

- Bump sensors

- Distance sensors

- Cliff sensors

### Decision Rules

IF obstacle detected

THEN turn

IF stairs detected

THEN reverse

### Action

The vacuum immediately changes direction.

Reactive navigation allows the vacuum to operate effectively even though it may not have a complete map of the home.

## Real-World Example 2: Automatic Emergency Braking Systems

Modern vehicles often contain collision-avoidance systems.

### Sensor Input

- Radar

- Cameras

- LIDAR

### Decision Rule

IF collision risk is high

THEN apply brakes

### Action

The vehicle automatically slows down or stops.

This is a reactive behavior because the action occurs immediately in response to sensor data.

## Advantages of Reactive Navigation

Reactive systems are:

- Fast

- Simple to program

- Reliable in changing environments

- Computationally efficient

Because of these advantages, reactive behaviors are often the first autonomous behaviors programmed on VEX robots.

## Limitations of Reactive Navigation

Reactive systems also have limitations:

- No long-term planning

- No memory of previous obstacles

- Can become trapped in corners or loops

- May not find the shortest path to a goal

For this reason, advanced robots often combine reactive behaviors with path planning and mapping algorithms.

## Multiple-Choice Question 1

A VEX AIM robot is driving forward when its Distance Sensor detects a wall 5 cm away. The robot immediately turns left to avoid a collision. Which robotics concept is being demonstrated?

A. Inverse Kinematics

B. Reactive Navigation ✅

C. Odometry

D. Path Planning

Correct Answer: B

Explanation: The robot is reacting directly to sensor input without first building a map or calculating a complex route.

## Multiple-Choice Question 2

Which of the following is the best example of a reactive behavior?

A. A warehouse robot calculating the shortest route across a building.

B. A self-driving car creating a city-wide navigation plan.

C. A robotic vacuum turning immediately after hitting a chair leg. ✅

D. A robot storing a map of an environment for future use.

Correct Answer: C

Explanation: Reactive behaviors are immediate responses to sensor inputs. Turning after detecting an obstacle is a classic example of reactive navigation.

## CodeLens Activity: VEX AIM Obstacle Avoidance

Read the following pseudocode and determine what the robot will do.

while True:

distance = DistanceSensor.object_distance()

if distance < 15:

turn_right(90)

else:

drive_forward()

### Question

If the robot detects an obstacle 10 cm away, what action will occur?

- A. Continue driving forward

- B. Stop permanently

- C. Turn right 90 degrees ✅

- D. Drive backward

Answer: C

Because the measured distance is less than 15 cm, the robot executes the obstacle-avoidance behavior and turns right.