## 4.1 Global Map Representations

Before a robot can calculate a path across a room, drive to a target, or navigate around obstacles, it must first create a digital representation of the physical world. Computers cannot directly understand walls, chairs, or game elements the way humans do. Instead, robots convert continuous physical space into a structured data format that navigation algorithms can process mathematically.

One of the most common map representations used in robotics is the occupancy grid map.

### Occupancy Grid Maps

An occupancy grid divides the environment into a collection of small square cells, similar to pixels in an image. Each cell stores information about whether that location is free, occupied, or unknown.

| Value | Meaning |
| --- | --- |
| 0 | Free space |
| 1 | Occupied space (obstacle) |
| -1 or 0.5 | Unknown or unexplored space |

Example:

[0][0][1][0][0]

[0][0][1][0][0]

[0][0][0][0][0]

[1][1][1][0][0]

In this map:

- The robot can travel through cells containing 0.

- Cells containing 1 represent obstacles.

- Unknown cells require additional exploration before they can be safely traversed.

Visually, an occupancy grid often appears like a black-and-white image:

□ □ ■ □ □

□ □ ■ □ □

□ □ □ □ □

■ ■ ■ □ □

Where:

- □ = free space

- ■ = obstacle

## Why Robots Use Grids

The real world contains an infinite number of positions.

For example, a robot could be located at:

(2.1, 3.4)

(2.11, 3.41)

(2.111, 3.411)

This creates an enormous number of possible locations.

To simplify navigation, robots divide the world into a finite number of cells.

This allows algorithms to answer questions such as:

- Can I move into this location?

- Is there an obstacle here?

- What is the shortest route to the goal?

## Converting a Grid into a Graph

Once an environment is represented as a grid, it can be transformed into a mathematical graph.

Each traversable cell becomes a node.

Adjacent cells become edges.

O --- O --- O

| | |

O --- O --- O

| | |

O --- O --- O

`

This representation allows robots to apply graph-search algorithms such as:

- Breadth-First Search (BFS)

- Dijkstra's Algorithm

- A*

- D*

These algorithms help the robot identify efficient routes through an environment.

# VEX AIM Example: Classroom Navigation

Imagine a VEX AIM robot operating inside a classroom.

The robot's goal is to drive from the doorway to a colored cube located across the room.

### Mapping the Environment

The classroom is divided into square cells:

□ □ □ □ □

□ ■ ■ □ □

□ □ □ □ □

□ □ ■ □ G

S □ □ □ □

Where:

- S = Start position

- G = Goal

- ■ = Desk or obstacle

The AIM robot uses:

- Distance Sensor

- Optical Sensor

- GPS-like position estimates

to determine which cells are occupied.

### Path Planning

Rather than driving directly toward the goal and hitting desks, the robot calculates a route through free cells.

A path-planning algorithm might choose:

S → → ↑ ↑ → → G

This route avoids collisions and guides the robot safely to its destination.

# VEX AIM Example: Competition Field Navigation

Suppose a VEX AIM robot is collecting game objects.

The field contains:

- Walls

- Field barriers

- Other robots

- Scoring zones

The robot can create a digital occupancy grid:

□ □ □ □ G

□ ■ ■ □ □

□ □ □ □ □

□ □ ■ □ □

S □ □ □ □

By converting the field into a graph, the AIM robot can determine:

- The shortest route to a game piece

- Which areas are blocked

- Alternate routes if another robot blocks the field

This is the same concept used by many industrial and research robots.

# Real-World Example 1: Robotic Vacuum Cleaner

Modern robotic vacuum cleaners often use occupancy grids to remember the layout of a home.

### Mapping

The robot scans:

- Walls

- Furniture

- Hallways

- Doorways

and converts them into a grid map.

Living Room = Open Cells

Sofa = Occupied Cells

Coffee Table = Occupied Cells

### Navigation

Once mapped, the vacuum can:

- Clean efficiently

- Avoid repeatedly visiting the same area

- Return to the charging dock

Without a map, the vacuum would simply wander randomly.

# Real-World Example 2: Autonomous Warehouse Robots

Warehouse robots operate inside highly organized environments containing:

- Shelves

- Workers

- Loading stations

- Other robots

These robots create occupancy grids of the warehouse.

Shelf = Occupied

Aisle = Free Space

Loading Dock = Goal

The grid becomes the foundation of the robot's navigation system.

Path-planning algorithms then determine:

- The shortest route

- The fastest route

- Safe routes around temporary obstacles

This enables thousands of warehouse robots to move efficiently throughout a facility.

# Real-World Example 3: Self-Driving Cars

Autonomous vehicles continuously generate occupancy maps of the environment.

Objects such as:

- Vehicles

- Pedestrians

- Cyclists

- Construction barriers

are represented as occupied regions.

Free road space becomes traversable cells.

The vehicle then uses these maps to determine:

- Where it can safely drive

- Which lanes are blocked

- How to navigate intersections

While modern self-driving cars often use more sophisticated map representations than simple occupancy grids, the underlying concept remains similar: convert the physical world into a digital representation that can be searched mathematically.

# Real-World Example 4: Mars Rovers

NASA's Mars rovers use occupancy-style maps to navigate unknown terrain.

The rover classifies terrain into:

- Traversable regions

- Rocks

- Cliffs

- Hazardous slopes

The resulting map allows the rover to plan safe routes without human drivers.

This is particularly important because communication delays between Earth and Mars make real-time remote control impossible.

## Advantages of Occupancy Grids

✅ Easy to understand

✅ Easy to visualize

✅ Compatible with path-planning algorithms

✅ Works with many sensor types

✅ Widely used in robotics

## Limitations of Occupancy Grids

❌ Large maps require significant memory

❌ Small cells increase computational cost

❌ Dynamic obstacles require frequent updates

❌ Fine details may be lost with large cell sizes

### Connection to VEX AIM and Modern Robotics

Whether a robot is:

- A VEX AIM robot navigating a classroom,

- A robotic vacuum cleaning a house,

- A warehouse robot moving inventory,

- A Mars rover exploring another planet,

the first step is often the same:

Convert the physical environment into a digital map.

Occupancy grids provide a simple and powerful way to transform real-world space into a mathematical structure that robots can understand, search, and navigate.

### Multiple-Choice Question: Occupancy Grid Maps

A VEX AIM robot creates the following occupancy grid while navigating a classroom:

□ □ □ □ G

□ ■ ■ □ □

□ □ □ □ □

□ □ ■ □ □

S □ □ □ □

Where:

- S = Start position

- G = Goal position

- □ = Free space

- ■ = Occupied cell (obstacle)

What is the primary purpose of representing the environment as an occupancy grid?

A. To increase the robot's motor speed.

B. To convert the physical environment into a digital map that can be used by path-planning algorithms. ✅

C. To eliminate the need for sensors.

D. To ensure the robot always travels in a straight line.

Correct Answer: B

Explanation: Occupancy grids allow robots to represent the physical world as a structured data set. Each cell stores information about whether the space is free, occupied, or unknown. Navigation algorithms such as A* and Dijkstra's Algorithm can then search this map to find safe routes to a destination.

### Multiple-Choice Question: Nodes and Edges

When an occupancy grid is converted into a graph for path planning, what do the free cells represent?

A. Obstacles

B. Motor commands

C. Nodes in the graph ✅

D. Sensor readings

Correct Answer: C

Explanation: In graph-based navigation, each traversable (free) cell becomes a node, while neighboring cells are connected by edges. This graph structure enables path-planning algorithms to calculate routes through the environment.

### Application Question

A robotic vacuum cleaner has mapped a living room using an occupancy grid. A chair is represented as a group of occupied cells. How will a path-planning algorithm treat those cells?

A. As preferred travel locations.

B. As charging stations.

C. As nodes that should be avoided when calculating a route. ✅

D. As free space.

Correct Answer: C

Explanation: Occupied cells represent obstacles such as furniture, walls, or barriers. Path-planning algorithms avoid these cells when computing safe routes through the environment.

### CodeLens Activity: Occupancy Grid Path Planning

Consider the following occupancy grid used by a VEX AIM robot:

0 = Free Space

1 = Obstacle

Grid:

[ S ][ 0 ][ 0 ][ 1 ][ G ]

[ 0 ][ 1 ][ 0 ][ 1 ][ 0 ]

[ 0 ][ 1 ][ 0 ][ 0 ][ 0 ]

[ 0 ][ 0 ][ 0 ][ 1 ][ 0 ]

The robot can move:

- North (↑)

- South (↓)

- East (→)

- West (←)

but cannot move through obstacles (1).

### Pseudocode

if grid[next_row][next_col] == 0:

move_to(next_row, next_col)

else:

find_alternate_path()

### Question

The VEX AIM robot is currently at position S and wants to move one cell to the right.

The cell to the right contains:

0

What will the robot do?

A. Stop because all movement requires path planning.

B. Move into the cell because it is free space. ✅

C. Treat the cell as an obstacle.

D. Return to the starting position.

Correct Answer: B

Explanation:

The pseudocode checks whether the next cell contains a 0 (free space). Since the cell to the right of the robot is free, the robot moves into that cell.

### Challenge CodeLens Activity

Consider the following occupancy grid:

[ S ][ 0 ][ 1 ][ G ]

[ 0 ][ 0 ][ 1 ][ 0 ]

[ 1 ][ 0 ][ 0 ][ 0 ]

The robot executes:

if front_cell == 1:

turn_right()

find_new_path()

else:

drive_forward()

### Question

The robot is facing the goal and detects that the cell directly in front contains a 1.

What happens next?

A. The robot drives through the obstacle.

B. The robot immediately reaches the goal.

C. The robot turns right and searches for an alternate route. ✅

D. The robot shuts down.

Correct Answer: C

Explanation:

In an occupancy grid, a value of 1 represents an occupied cell. Because the front cell is blocked, the robot cannot move forward and must find an alternate path around the obstacle. This is the same behavior a VEX AIM robot would use when navigating around barriers or classroom obstacles.