## Chapter 1: Introduction to Robotic Subsystems & Control Loop

Welcome to your first workbook chapter! Before diving into programming your physical VEX robot, we must understand the fundamental software patterns that dictate how a robot processes information and interacts with its environment.

### 1.1 The Core Paradigms of Robotics

At its core, any autonomous robotic system can be broken down into a repeating operational cycle known as the Sense-Plan-Act paradigm (also widely referred to as the Perception-Planning-Control loop).

[ Perception (Sense) ] ----> [ Planning (Plan) ] ----> [ Control (Act)           ^                                        |_________________ Environment Feedback ______________|

- Perception (Sense): The robot gathers raw data from its environment using physical hardware sensors and processes it into meaningful context.

- Planning (Plan): The robot processes the parsed information, evaluates its mission objectives, and determines what action it should take next.

- Control (Act): The structural hardware signals are sent to the actuators (motors, pistons, etc.) to physically execute the planned action in the real world.

### 1.2 Control System Foundations

To make a robot move reliably, software engineers utilize two main types of mathematical frameworks:

#### Open-Loop Control Systems

An open-loop control system computes its input signals using only current state estimations and pre-determined models, without utilizing any live data from sensors to correct its trajectory.

Example: Telling your VEX drivetrain to run its motors at 50% power for exactly 3 seconds. If an obstacle gets in the way or the floor is slippery, the robot will fail to reach its destination because it cannot adjust dynamically.

#### 1.3 Closed-Loop Control Systems

A closed-loop control system constantly measures the active output of the system via sensors and feeds that live state data back into the controller. The system compares the actual state against a desired target state (or setpoint) to calculate an error value.

Error=Setpoint-Current State

The system then continuously adjusts its actuator power outputs to minimize this error to 0.

#### PID Control Fundamentals

The most universally implemented closed-loop methodology in robotics is Proportional-Integral-Derivative (PID) control. While a full PID controller tracks past errors (Integral) and future error trajectories (Derivative), we will begin by isolating the foundational Proportional Control (P-Control) mechanism.

In a basic proportional control loop, the system output (such as motor voltage) is calculated by multiplying the active error by a customizable scaling constant called the proportional gain (Kp):

Output=Kp×Error

If your Kp value is tuned poorly, the system might not receive enough power to overcome friction as it nears the goal, resulting in steady-state error—a constant, persistent gap where the robot stalls just short of its intended target.

### 1.4 Sensors and Actuators

### 1.5 ActiveCode Exercise: 1D Proportional Control Loop Simulation

Run and modify the Python code block below. This simulates a 1-dimensional robot trying to reach a designated goal position (xgoal=10.0).

- Observe what happens with a weak gain (Kp=0.3).

- Notice how friction causes a steady-state error.

- Try tuning Kp higher (e.g., 1.5 or 2.5) to see how it alters the robot's ability to settle precisely on the setpoint.

Python

# ActiveCode Simulation

import time

def simulate_p_control(Kp, target_pos=10.0):

current_pos = 0.0

time_steps = 20

friction_threshold = 0.5 # Minimum power required to move the robot

print(f"Starting Simulation with Kp = {Kp}")

print("---------------------------------------")

for step in range(1, time_steps + 1):

# 1. Calculate Error

error = target_pos - current_pos

# 2. Compute Proportional Output

motor_output = Kp * error

# Simulated physics: If motor power is too weak, friction stops it completely

if abs(motor_output) < friction_threshold:

actual_movement = 0.0

else:

actual_movement = motor_output * 0.25 # scaling time step movement

current_pos += actual_movement

print(f"Step {step:02d} | Position: {current_pos:.2f} | Error: {error:.2f} | Motor Power: {motor_output:.2f}")

if abs(error) < 0.01:

print(f"\nTarget successfully reached in {step} steps!")

return

print(f"\nSimulation concluded. Final Steady-State Error: {target_pos - current_pos:.2f}")

# TODO: Run this, then change Kp to 1.8 and run it again!

simulate_p_control(Kp=0.3)

### 1.4 Conceptual Check

Question:

Imagine you are building your VEX Robot to complete the upcoming sports ball navigation challenge. You mount a front-facing mechanical bumper switch to detect walls and a heavy-duty drivetrain to handle wheel rotation.

Map each hardware element to its correct functional phase inside the standard Perception-Planning-Control loop.

- The bumper switch registering a click sequence when hitting a barrel.

- An if/else statement deciding to turn left because the front bumper is pressed.

- Sending electrical current commands to the left and right wheel motors to execute the spin.

Select the correct structural mapping order:

- (A) Plan → Act → Sense

- (B) Perception → Planning → Control

- (C) Control → Perception → Planning

- (D) Perception → Control → Planning

Check Answer: The correct answer is (B). Reading physical sensor interactions represents Perception, processing conditional behavior logic represents Planning, and driving physical motor systems represents Control.