## Chapter 11: Multi-Node Coordination & Closed-Loop Simulation

In previous chapters, we looked at how isolated ROS2 nodes talk to one another through simple asynchronous data pipelines. Now, we will bring everything together to see how an entire collection of specialized nodes runs simultaneously to solve high-level tasks. This chapter covers multi-node coordination, structural frame transformations using rotation matrices, and utilizing TurtleSim to model and simulate real-world differential drive kinematics.

### 11.1 Managing Distributed Robotics Architecture

A complete autonomous system runs multiple processes at the same time to control the robot:

[ Vision Processing Node ] ──( /detected_object )──> [ Motion Control Node ]

│

( /cmd_vel Topic )

▼

[ TurtleSim Simulator ]

Coordinating a modular pipeline like this introduces unique failure points. If one component stops working, the entire system can break:

- Mismatched Data Types: A classic system integration bug happens when a developer sets up a publisher node to send a customized coordinate type while the subscriber expects a standard geometry package. ROS2 blocks these mismatched topics immediately at the graph level to prevent errors.

- Missing Node Paths: If the vision node crashes or drops off the graph, the controller node will stop receiving data. Without an internal fallback rule, the robot will continue executing its last received motion command, leading to crashes.

### 11.2 Coordinate System Transformations & Rotation Matrices

When a robot tracks items in its environment, its vision sensor measures objects in a local sensor coordinate frame relative to the camera face. However, to navigate or update a map, the motion controller must translate those targets into a global world coordinate frame.

To transform a 2D position vector from a local frame to a global frame rotated by the robot's heading angle (θ), we use a standard 2D Rotation Matrix, denoted as Rθ:

Rθ=cosθ-sinθsinθcosθ

Multiplying a local relative coordinate vector plocal=xlocal,ylocalT by this rotation matrix gives us the aligned global position vector pglobal relative to the robot's position:

xglobal=xlocalcosθ-ylocalsinθ

yglobal=xlocalsinθ+ylocalcosθ

### 11.3 ActiveCode Exercise: Local-to-Global Coordinate Transformation

Complete the Python script below. Fill in the transformation math using the 2D rotation matrix formulas to convert an object's relative coordinate position into global map coordinates.

Python

import math

def transform_local_to_global(local_x, local_y, robot_x, robot_y, robot_theta):

"""

Transforms a target position from the robot's local sensor frame

into the global simulator map coordinate frame using a 2D rotation matrix.

"""

# 1. TODO: Apply the rotation matrix formulas to align the local vector

rotated_x = local_x * math.cos(robot_theta) - local_y * math.sin(robot_theta)

rotated_y = local_x * math.sin(robot_theta) + local_y * math.cos(robot_theta)

# 2. TODO: Shift the rotated coordinates by the robot's global position offset

global_x = rotated_x + robot_x

global_y = rotated_y + robot_y

return global_x, global_y

# Test Scenario: Robot is at global (x=5.0, y=2.0) facing 90 degrees (pi/2 radians)

# The local sensor spots a soccer ball exactly 2.0 meters directly in front of it (local_x=2.0, local_y=0.0)

r_x, r_y = 5.0, 2.0

r_theta = math.pi / 2.0  # Facing straight up along the Y-axis

g_x, g_y = transform_local_to_global(local_x=2.0, local_y=0.0, robot_x=r_x, robot_y=r_y, robot_theta=r_theta)

print("--- Frame Transformation Matrix ---")

print(f"Robot Global Position: ({r_x}, {r_y}) at {math.degrees(r_theta)}°")

print(f"Detected Object Global Coordinates: ({g_x:.2f}, {g_y:.2f})")

### 11.4 Conceptual Check

Question: While launching your multi-node autonomous configuration for your final application lab prep, your team notices that the simulated robot sits completely idle. When you inspect the active graph terminal, you verify that:

- The camera vision node is running and publishing coordinates to /vision_data using custom String messages.

- The movement planning node is running and listening to a topic named /vision_data, but it expects incoming packets to be structured as geometry_msgs/Point types.

Why is the communication connection failing to share data?

- (A) The system is trapped in a local minimum equilibrium zone.

- (B) The distance sensor array suffered an uncorrected odometer drift.

- (C) The publisher and subscriber nodes are bound to conflicting topic message types, preventing the ROS2 graph from delivering the data packet stream.

- (D) The transformation rotation matrix parameters were calculated using degrees instead of radians.

Check Answer: The correct answer is (C). For communication to work in ROS2, any subscriber listening to a topic must match the exact data type definition used by the publisher node on that same channel. Because one node uses text strings while the other expects structural coordinate points, the framework blocks the data pipeline.