## Chapter 2: Robot Kinematics & Coordinate Spaces

Now that we understand how a control loop works, we need to explore how a robot calculates its actual position and movement in a physical environment. This chapter focuses on the geometric math behind wheeled movement, allowing you to transition smoothly into your physical navigation labs.

### 2.1 Kinematic Configurations & Differential Drive

In robotics, kinematics is the study of motion without considering the forces that cause it. It allows us to calculate exactly how a robot will move based on the geometry and speed of its wheels.

The VEX robot utilizes a standard differential drive platform. A differential drive robot consists of two independent, parallel wheels mounted on a common axis, with one or more passive casters supporting its weight. Because the wheels are independent, the robot changes direction by varying the relative rate of rotation between the left and right motors rather than relying on a traditional steering wheel axle.

To calculate these movements, we must analyze the robot's dimensions using its track width (L)—the measured distance between the center point of the left wheel contact patch and the center point of the right wheel contact patch.

### 2.2 Forward vs. Inverse Kinematics

When working with mobile robots, you will constantly move back and forth between two distinct mathematical mappings:

[ Wheel Velocities: v_L, v_R ] === Forward Kinematics ===> [ Robot Path: v, ω ]

[ Wheel Velocities: v_L, v_R ] <=== Inverse Kinematics === [ Robot Path: v, ω ]

#### Forward Kinematics

Forward kinematics calculates the overall robot velocities (linear velocity v and angular velocity ω) based on the known, individual speeds of the wheels (vL and vR).

The average linear velocity (v) across the center point of the robot is given by:

v=vR+vL2

The angular velocity (ω) around the center point, measured in radians per second, is determined by the speed difference divided by the track width (L):

ω=vR-vLL

To approximate the displacement changes (Δx,Δy,Δθ) over a short time step (Δt), we can project these velocities relative to the robot's current heading angle (θ):

Δθ=ω×Δt

Δx=v×cosθ×Δt

Δy=v×sinθ×Δt

#### Inverse Kinematics

Inverse kinematics does the exact opposite: it takes a desired target path velocity (how fast you want the overall robot to move forward (v) and spin (ω)) and breaks it down into the specific target wheel velocities (vL,vR) required to execute that motion.

Rearranging our earlier equations gives us the definitive formulas for inverse kinematics:

vL=v-ω×L2

vR=v+ω×L2

### 2.3 ActiveCode Exercise: Differential Drive Forward Kinematics

Complete the missing formulas inside the function below. Given the left wheel speed, right wheel speed, track width L, and the current heading angle theta, calculate the resulting changes in coordinate space.

Python

import math

def calculate_forward_kinematics(v_L, v_R, L, theta, dt=1.0):

# TODO: Implement the linear velocity formula

v = (v_R + v_L) / 2.0

# TODO: Implement the angular velocity formula

omega = (v_R - v_L) / L

# Calculate coordinate changes

delta_theta = omega * dt

delta_x = v * math.cos(theta) * dt

delta_y = v * math.sin(theta) * dt

return delta_x, delta_y, delta_theta

# Test Case: Robot heading straight along the X axis (theta = 0)

# Right wheel turning faster than left wheel should cause it to pivot left

dx, dy, dtheta = calculate_forward_kinematics(v_L=2.0, v_R=4.0, L=0.5, theta=0.0)

print("--- Kinematics Output ---")

print(f"Delta X: {dx:.2f} meters")

print(f"Delta Y: {dy:.2f} meters")

print(f"Delta Theta (Radians): {dtheta:.2f}")

### 2.4 Parsons Problem: Circular Trajectory Inverse Kinematics

Challenge: Drag and drop the blocks of Python code into the correct logical sequence to build a function that calculates the required wheel speeds (vL,vR) to make a differential drive robot drive in a perfect, continuous circle of radius R at a target linear speed v.

Hint: Angular velocity is defined as ω=vR.

# Block 1

def compute_circular_wheel_speeds(v, R, L):

# Block 2

omega = v / R

# Block 3

v_L = v - (omega * L) / 2.0

v_R = v + (omega * L) / 2.0

# Block 4

return v_L, v_R

### 2.5 Conceptual Check

Question: Imagine you are programming your VEX robot for Lab 2 to navigate around a series of tight corners. If your robot's track width (L) is exactly 0.3 meters, and you command both the left wheel (vL) and right wheel (vR) to spin forward at a matching speed of 1.5 meters per second, what is the resulting angular velocity (ω) of the robot?

- (A) 1.0 rad/s

- (B) 5.0 rad/s

- (C) 0.0 rad/s

- (D) 0.45 rad/s

Check Answer: The correct answer is (C). Because vR and vL are equal (1.5-1.5=0), the numerator for our angular velocity equation becomes zero, meaning the robot moves in a perfectly straight line without spinning at all.