## Chapter 4: Map Representations & Path Planning (A*)

In previous chapters, we focused on how a robot tracks its movements and avoids immediate local obstacles. However, for a robot to complete complex, high-level objectives—such as navigating to the opponent's goal in a soccer match—it must look at the bigger picture. This chapter covers how robots represent environments globally and compute optimal routes using the standard A* pathfinding algorithm.

### 4.1 Global Map Representations

Before a robot can calculate a path across a room, it must convert continuous physical space into a discrete digital data structure that an algorithm can parse.

The most common approach in mobile robotics is an occupancy grid map. An occupancy grid discretizes a 2D environment into a matrix of uniform, pixel-like cells. Each cell contains a value representing the probability that the space is physically occupied by an obstacle:

- 0 (White): Free space where the robot can safely traverse.

- 1 (Black): Occupied space containing a wall, barrier, or field boundary.

- -1 or 0.5 (Gray): Unknown or unexplored space.

[0][0][1][0][0]

[0][0][1][0][0]  <-- A 2D Matrix representing an Occupancy Grid

[0][0][0][0][0]      where '1' indicates a solid obstacle block.

[1][1][1][0][0]

By viewing the map as a grid, we can model it as a formal mathematical graph. Each free cell acts as a node, and the transitions to adjacent cells (North, South, East, West) serve as the edges connecting them.

### 4.2 The A* Pathfinding Algorithm

The A* Pathfinding Algorithm is an informed heuristic search algorithm designed to find the shortest path through a graph network from a designated starting node to a goal node.

A* evaluates nodes by combining two distinct cost metrics into a single fitness score, fn:

fn=gn+hn

- gn (Path Cost): The exact cost or distance traveled from the start node to the current node n.

- hn (Heuristic Cost): An estimated guess of the remaining distance from the current node n to the final goal. To ensure the algorithm works correctly, the heuristic must be admissible, meaning it never overestimates the actual remaining cost.

In grid maps where diagonal movement is restricted, the most common admissible heuristic is the Manhattan distance:

hn=xgoal-xn+ygoal-yn

### 4.3 ActiveCode Exercise: Completing the A* Scoring System

Below is a partial Python implementation of the A* node evaluation loop. Your task is to fill in the missing code lines to properly compute the path cost gn, the Manhattan distance heuristic hn, and the total combined tracking score fn.

Python

# ActiveCode Exercise: A* Node Scoring

def evaluate_node(current_x, current_y, neighbor_x, neighbor_y, goal_x, goal_y, parent_g):

"""

Calculates the A* scoring metrics for a neighboring node.

Assumes moving to an adjacent cell costs exactly 1.0 unit.

"""

# 1. Calculate g(n): The parent node's path cost plus the cost to move to the neighbor

# TODO: Complete the line below

g_cost = parent_g + 1.0

# 2. Calculate h(n): The Manhattan distance heuristic to the final goal

# TODO: Complete the line below

h_cost = abs(goal_x - neighbor_x) + abs(goal_y - neighbor_y)

# 3. Calculate f(n): The total cost function

# TODO: Complete the line below

f_cost = g_cost + h_cost

return g_cost, h_cost, f_cost

# Test Scenario

start_g = 5.0  # The robot has already traveled 5 units to get here

current = (2, 3)

neighbor = (2, 4)

goal = (5, 7)

g, h, f = evaluate_node(current[0], current[1], neighbor[0], neighbor[1], goal[0], goal[1], start_g)

print("--- A* Evaluation Matrix ---")

print(f"Path Cost g(n): {g:.1f}")

print(f"Heuristic Cost h(n): {h:.1f}")

print(f"Total Cost f(n): {f:.1f}")

### 4.4 Parsons Problem: The A* Main Loop Sequence

Challenge: Arrange the core operational steps of the main A* execution loop into the correct logical order from top to bottom.

# Block 1

while open_list_is_not_empty():

# Block 2

current_node = pop_node_with_lowest_f_score(open_list)

add_to_closed_list(current_node)

# Block 3

if current_node == goal_node:

return reconstruct_optimal_path()

# Block 4

for neighbor in get_valid_neighbors(current_node):

if neighbor in closed_list:

continue

calculate_and_update_node_costs(neighbor)

push_to_open_list_if_new(neighbor)

### 4.5 Conceptual Check

Question: While observing your robot run an A* pathfinding algorithm on an occupancy grid, you notice that it completely ignores a wide-open path and chooses a longer route around the edge of the arena. What bug in your code would cause the A* search behavior to behave like an inefficient, unguided search?

- (A) The grid cell coordinate index system was inverted horizontally.

- (B) The heuristic function hn is returning a constant value of 0 for every node, turning A* into Dijkstra’s Algorithm.

- (C) The path cost function gn is growing exponentially faster than the physical step metric layout.

- (D) The target goal node coordinates were mapped outside the array boundaries.

Check Answer: The correct answer is (B). When the heuristic hn is set to 0, the algorithm relies entirely on the accumulated path cost gn. This strips away the directional guidance of A*, causing it to search outward in all directions like a standard Dijkstra search.