## Chapter 7: End-to-End Autonomous Driving

In Chapter 6, we discovered how computer vision lets a robot detect specific visual features like bounding boxes and color thresholds. Typically, a robot uses those features to calculate its relative distance to an object and then passes that distance to a separate planning module. However, modern autonomous vehicles often skip these intermediate geometric calculations entirely. This chapter covers end-to-end autonomous driving, where a robot learn a direct mapping from raw visual sensor inputs straight to motor control command outputs.

### 7.1 Behavioral Cloning in Robotics

End-to-end learning simplifies the traditional robotics pipeline by replacing separate perception, planning, and control modules with a single integrated model. In the context of autonomous navigation, this is usually achieved through a technique called behavioral cloning—a form of supervised learning where an AI agent learns to navigate by mimicking the driving behaviors of a human expert.

Traditional:  [Vision Sensor] ──> [Object Detection] ──> [Path Planner] ──> [Motor Control]

End-to-End:   [Vision Sensor] ───────────────────> [ ML Model ] ───────────────────> [Motor Control]

During the data collection phase, a human operator manually drives the robot along a target track. The system simultaneously logs two synchronized data streams at every time step:

- Features (X): The raw sensory view captured by the camera or vision sensor.

- Labels (y): The exact control actions executed by the human driver at that exact moment, such as the target linear velocity (v) and angular velocity (ω) or steering angle.

An optimization model is then trained on these pairs to reproduce the expert's driving commands given similar visual scenes.

### 7.2 Training Loops & Regression Evaluation

Because steering a robot involves selecting a value along a continuous spectrum (e.g., steering anywhere from -30∘ to +30∘), behavioral cloning operates as a regression task.

To determine how accurately our model has cloned the expert's behavior during a training loop, we must evaluate its performance mathematically. The standard metric used to calculate this error is the Mean Squared Error (MSE). MSE measures the average squared difference between the model's predicted steering values and the true expert labels across the dataset:

MSE=1ni=1nyi-yi2

Where:

- yi is the actual target command provided by the expert driver.

- yi is the command predicted by the autonomous driving network.

- n is the total number of data samples evaluated.

### 7.3 Dataset Bias & Generalization Challenges

While end-to-end systems can handle highly complex environments, they suffer from a major vulnerability: dataset bias. A machine learning model can only learn from the specific data it is given. If the training data lacks variety, the model will struggle to adjust to new situations.

The Recovery Problem: Imagine an expert driver who navigates a track perfectly down the center line. Because the driver never makes a mistake, the dataset only contains images of a perfectly centered track paired with straight-forward steering commands. If the autonomous robot is deployed and drifts slightly off course due to wheel slippage, it will encounter a completely unfamiliar visual perspective. Because it has never trained on "recovery data" showing how to steer back from the edge, the model will fail to generalize, often driving completely off the track.

### 7.4 ActiveCode Exercise: Calculating Steering Regression Loss

Run and complete the Python evaluation script below. This code calculates the overall Mean Squared Error (MSE) for a small batch of steering predictions generated by a prototype end-to-end autonomous model.

Python

def calculate_steering_mse(expert_commands, model_predictions):

"""

Calculates the Mean Squared Error between the expert's true steering angles

and the model's predicted autonomous outputs.

Both inputs are lists of floats representing angles in radians.

"""

total_squared_error = 0.0

num_samples = len(expert_commands)

# Loop through the paired datasets

for i in range(num_samples):

true_angle = expert_commands[i]

predicted_angle = model_predictions[i]

# TODO: Calculate the difference, square it, and add to the total

error = true_angle - predicted_angle

squared_error = error ** 2

total_squared_error += squared_error

# Calculate the mean

mse = total_squared_error / num_samples

return mse

# Evaluation Data: Steering commands in radians for 4 consecutive frames

expert_targets = [0.00,  0.34, -0.12,  0.58]

model_outputs  = [0.05,  0.28, -0.20,  0.42]

calculated_loss = calculate_steering_mse(expert_targets, model_outputs)

print("--- Model Performance Metrics ---")

print(f"Evaluated Samples: {len(expert_targets)}")

print(f"Calculated Steering MSE Loss: {calculated_loss:.5f}")

### 7.5 Conceptual Check

Question: You are training your VEX robot for Lab 5 (End-to-End autonomous driving). You collect all your training data by driving the robot clockwise around a symmetrical, right-turning circular track. When you deploy the trained model autonomously, the robot successfully drives in a clockwise circle. However, when you place the robot on a counter-clockwise track requiring a left turn, it immediately drives straight into the wall. What is the most likely cause of this failure?

- (A) The robot's inverse kinematics matrix inverted its angular velocity inputs automatically.

- (B) The model is suffering from severe dataset bias because the training dataset contained no examples of left-turning visual features or negative angular velocity commands.

- (C) The Mean Squared Error of the training loop converged directly to zero.

- (D) The AI vision sensor automatically defaults to a closed-loop P-control mode when encountering left walls.

Check Answer: The correct answer is (B). Because the model was trained exclusively on right turns, it has no data showing how to process left-turning visual fields. It cannot generalize to a left turn because it has never encountered those features or actions in its training dataset.