## Chapter 8: Multi-Modal Integration & State Machines

Up to this point, we have treated robotics tasks as isolated problems: driving to coordinates, avoiding a single obstacle, or recognizing an object via an AI vision sensor. However, real-world robotic tasks require a robot to execute sequence-dependent behaviors. For instance, in Lab 6 (Cargo Sorting), a robot must search for an object, navigate toward it, grab it, identify its target destination, deposit it, and reset. To coordinate these multi-step operations reliably without getting stuck, engineers use structured decision frameworks.

### 8.1 Finite State Machines (FSM)

A Finite State Machine (FSM) is a mathematical model of computation used to design robotic behavior sequences. An FSM consists of a fixed number of discrete operational conditions called states, but the system can only occupy exactly one state at any given moment (known as the current state).

The robot changes from one operational condition to another through a process called a transition. Transitions are initiated by specific conditions or sensory inputs known as triggers.

┌───────────┐

│   IDLE    │

└─────┬─────┘

│  Trigger: Start Button Pressed

▼

┌───────────────┐

┌───>│ SEEKING_CARGO ├─┐

│    └───────────────┘ │ Trigger: Cargo Detected in View

│                      ▼

│              ┌───────────────┐

│              │   GRABBING    │

│              └───────┬───────┘

│                      │ Trigger: Capture Grasp Confirmed

│                      ▼

│              ┌───────────────┐

│              │  DELIVERING   │

│              └───────┬───────┘

│                      │ Trigger: Cargo Released at Goal

└──────────────────────┘

#### FSM Architecture in Mission Planning

Using an FSM prevents a common programming error: complex nested conditional trees. Instead of writing massive, fragile loops packed with if/else checks, an FSM groups behaviors into modular states. This ensures that the robot only evaluates the sensors and controls relevant to its current step. For example, the robot will completely ignore drop-off zone markings while it is still actively searching for cargo on the open field.

### 8.2 State Coding Patterns using Enums

When writing an FSM in Python, engineers typically use the enum module to define explicit, human-readable states. Using an Enum (Enumeration) prevents typographical errors, as the compiler or interpreter will catch any misspelled state names immediately, rather than letting the robot fail silently at runtime.

### 8.3 ActiveCode Exercise: Coding a Mission Control FSM

Run and complete the Python state machine script below. The code models the foundational sequence for an autonomous warehouse robot handling states: IDLE, SEEKING_CARGO, GRABBING, and DELIVERING. Fill in the missing trigger evaluations inside the loop to complete the state transitions smoothly without creating infinite loops.

Python

from enum import Enum

class RobotState(Enum):

IDLE = 1

SEEKING_CARGO = 2

GRABBING = 3

DELIVERING = 4

def run_mission_loop(sensor_log):

# Set the initial state

current_state = RobotState.IDLE

print(f"Initial State: {current_state.name}")

print("---------------------------------------")

for cycle, sensors in enumerate(sensor_log, 1):

print(f"\n[Cycle {cycle}] Current State: {current_state.name}")

if current_state == RobotState.IDLE:

# TODO: Transition to SEEKING_CARGO if autonomous mode is enabled

if sensors.get("auto_enabled"):

current_state = RobotState.SEEKING_CARGO

print("-> Trigger: System activated. Scanning field...")

elif current_state == RobotState.SEEKING_CARGO:

# TODO: Transition to GRABBING if vision sensor detects cargo

if sensors.get("cargo_in_view"):

current_state = RobotState.GRABBING

print("-> Trigger: Target sighted! Engaging gripper subsystems.")

elif current_state == RobotState.GRABBING:

# TODO: Transition to DELIVERING if limit switches confirm secure hold

if sensors.get("grip_secured"):

current_state = RobotState.DELIVERING

print("-> Trigger: Object secured. Re-routing to drop-off station.")

elif current_state == RobotState.DELIVERING:

# TODO: Return to SEEKING_CARGO once cargo is successfully released

if sensors.get("cargo_released"):

current_state = RobotState.SEEKING_CARGO

print("-> Trigger: Delivery complete. Resetting exploration scan.")

# Simulated sensor readings over 5 runtime iterations

mock_sensors = [

{"auto_enabled": True,  "cargo_in_view": False, "grip_secured": False, "cargo_released": False},

{"auto_enabled": True,  "cargo_in_view": True,  "grip_secured": False, "cargo_released": False},

{"auto_enabled": True,  "cargo_in_view": True,  "grip_secured": True,  "cargo_released": False},

{"auto_enabled": True,  "cargo_in_view": False, "grip_secured": False, "cargo_released": True},

{"auto_enabled": True,  "cargo_in_view": False, "grip_secured": False, "cargo_released": False}

]

run_mission_loop(mock_sensors)

### 8.4 Parsons Problem: Color Profile Cargo Sorting Logic

Challenge: Arrange the conditional sorting checks below in the correct order to parse incoming cargo profiles by color. The machine must inspect the item's red value, direct it to the correct sorting bay, and then reset its scanner state.

# Block 1

if current_state == RobotState.INSPECTING_CARGO:

# Block 2

if color_sensor_red > 200:

target_bin = "RED_CARGO_BAY"

else:

target_bin = "GREEN_CARGO_BAY"

# Block 3

actuate_sorting_conveyor(target_bin)

# Block 4

current_state = RobotState.RESETTING_SCANNER

### 8.5 Conceptual Check

Question: During a trial run of Lab 6 (Cargo Sorting), your robot successfully approaches a block of cargo, closes its structural claw around it, but then stops moving completely. When debugging the code, you notice that the robot is stuck in the GRABBING state and never updates its state variable to DELIVERING. Which of the following software bugs explains this issue?

- (A) The system is trapped in a local minimum caused by conflicting potential field vectors.

- (B) The state transition trigger condition requires a grip_secured sensor flag to equal True, but the hardware limit switch inside the claw never closed fully to signal a successful grasp.

- (C) The odometer drift value grew exponentially, forcing the state machine to reset its state history.

- (D) The robot's differential drive kinematics equations inverted the angular velocity values while the arm was moving.

Check Answer: The correct answer is (B). In an FSM framework, a system cannot advance to the next state unless its explicit transition trigger criteria are met. If a sensor fails to register the required condition (e.g., a physical claw switch failing to click), the robot will remain stuck in its current state indefinitely.