### 1.3 Closed-Loop Control Systems

A closed-loop control system constantly measures the active output of the system via sensors and feeds that live state data back into the controller. The system compares the actual state against a desired target state (or setpoint) to calculate an error value.

Error=Setpoint-Current State

The system then continuously adjusts its actuator power outputs to minimize this error to zero.

#### Real-World Example 1: Automobile Cruise Control

Modern vehicles use closed-loop control to maintain a driver's selected speed.

Suppose a driver sets the cruise control to 65 miles per hour.

##### Sense

The vehicle continuously measures its actual speed using wheel-speed sensors.

##### Plan

The cruise control computer calculates the difference between the desired speed (65 mph) and the current speed.

For example:

Error=65-62=3 mph

The vehicle is traveling too slowly, so the controller determines that additional engine power is needed.

##### Act

The controller increases throttle input to accelerate the vehicle.

As the vehicle approaches the desired speed, the error becomes smaller and the controller reduces the amount of throttle applied.

If the vehicle begins climbing a hill, the speed drops and the controller automatically provides more power. If the vehicle travels downhill, the controller reduces power or may apply braking.

Without this feedback loop, the vehicle would constantly speed up and slow down as road conditions changed.

#### Real-World Example 2: Drone Altitude Control

Consumer and commercial drones rely heavily on closed-loop control systems to maintain stable flight.

Suppose a drone is commanded to hover at an altitude of 10 meters.

##### Sense

The drone uses:

- Altimeters

- GPS sensors

- Barometric pressure sensors

- Inertial Measurement Units (IMUs)

to determine its current height.

##### Plan

The flight controller continuously calculates:

Error=10 m-8.5 m=1.5 m

The drone is below its target altitude.

##### Act

The controller increases motor power, causing the propellers to generate more lift.

When the drone reaches 10 meters, the error approaches zero and motor outputs stabilize.

If a gust of wind pushes the drone downward, the control system immediately detects the change and applies corrective motor commands to return to the target altitude.

Without closed-loop feedback, the drone would be nearly impossible to hover accurately.

## PID Control Fundamentals

The most universally implemented closed-loop methodology in robotics is Proportional-Integral-Derivative (PID) control. While a full PID controller tracks past errors (Integral) and future error trajectories (Derivative), we will begin by isolating the foundational Proportional Control (P-Control) mechanism.

In a basic proportional control loop, the system output (such as motor voltage) is calculated by multiplying the active error by a customizable scaling constant called the proportional gain (Kp):

Output=Kp×Error

### Example: Robot Driving to a Wall

Suppose a robot is programmed to stop exactly 100 cm from a wall.

Current distance:

60 cm

Error:

100-60=40 cm

If:

Kp=0.5

then:

Output=0.5×40=20

The robot commands a large motor output because it is far from the target.

As the robot approaches the goal and reaches 95 cm:

100-95=5
Output=0.5×5=2.5

The motor power decreases because the error is much smaller.

This natural reduction in power helps produce smoother and more accurate movement.

### Understanding Steady-State Error

If your Kpvalue is tuned poorly, the system might not receive enough power to overcome friction as it nears the goal, resulting in steady-state error—a constant, persistent gap where the robot stalls just short of its intended target.

For example, a robotic arm attempting to rotate to 90° may stop at 87° because the remaining error is too small to generate sufficient motor torque. The arm remains close to the target but never actually reaches it.

This limitation is one reason why many real-world robotic systems incorporate the additional Integral and Derivative terms of PID control to improve accuracy and stability.

### Multiple-Choice Question: Closed-Loop Control

A robot is programmed to drive exactly 2 meters forward. It uses wheel encoders to continuously measure how far it has traveled and adjusts motor power until it reaches the target distance. What type of control system is the robot using?

A. Open-loop control because it follows a pre-programmed command only

B. Closed-loop control because it uses sensor feedback to correct its movement ✅

C. Manual control because the programmer specified a destination

D. Random control because the motor outputs change during operation

Correct Answer: B

Feedback:

- A: Incorrect. Open-loop systems do not use sensor feedback to verify performance.

- B: Correct. Closed-loop systems continuously compare the current state to the desired state and make corrections.

- C: Incorrect. The robot is operating autonomously, not manually.

- D: Incorrect. The motor outputs change based on measured error, not randomly.

Explanation:
Closed-loop control systems use sensor data to monitor performance and minimize error between the desired setpoint and the actual state.

### Multiple-Choice Question: PID Control

A robotic arm is attempting to move to a target position of 90°. It is currently at 70°. The controller uses the equation

Output=Kp×Error

If Kp=2, what output should the controller generate?

A. 20 ✅

B. 40

C. 140

D. 180

Correct Answer: A

Feedback:

- A: Correct. Error = 90 − 70 = 20. Therefore, Output = 2 × 20 = 40? Wait—check the calculation.

- B: Correct calculation. Error = 20, so Output = 2 × 20 = 40. ✅

- C: Incorrect. This value does not follow the proportional control equation.

- D: Incorrect. This is the target position, not the controller output.

Explanation:
First calculate the error:

Error=90-70=20

Then apply the proportional control equation:

Output=Kp×Error
Output=2×20=40

The controller should output 40 units of motor command.

Correct Answer: B

### Multiple-Choice Question: Understanding PID Benefits

Why do many robotics engineers use PID controllers instead of simple proportional control alone?

A. PID controllers eliminate the need for sensors.

B. PID controllers only work for robotic arms.

C. PID controllers can reduce steady-state error and improve system stability. ✅

D. PID controllers make robots move at maximum speed at all times.

Correct Answer: C

Feedback:

- A: Incorrect. PID controllers require feedback from sensors.

- B: Incorrect. PID controllers are used in many robotic systems, including drivetrains, drones, and industrial robots.

- C: Correct. The Integral and Derivative terms help improve accuracy and stability while reducing steady-state error.

- D: Incorrect. PID controllers adjust output based on error rather than always using maximum power.

Explanation:
PID control improves upon simple proportional control by using information about past errors (Integral) and predicted trends (Derivative) to achieve more accurate and stable performance.