## 2.1 Kinematic Configurations & Differential Drive

In robotics, kinematics is the study of motion without considering the forces that cause it. Rather than examining motor torque or friction, kinematics focuses on describing how a robot moves through space based on its geometry and wheel speeds.

Understanding kinematics allows engineers to predict:

- How far a robot will travel

- How quickly it will move

- How sharply it can turn

- Where it will be located after executing a command

Kinematic models are used in everything from educational VEX robots to autonomous vehicles and warehouse robots.

### Differential Drive Robots

The VEX robot utilizes a standard differential drive platform.

A differential drive robot consists of:

- Two independently powered wheels

- One or more passive caster wheels for balance

- Separate motors controlling the left and right sides

Unlike a car, which turns by changing the angle of its front wheels, a differential drive robot turns by changing the relative speeds of its left and right wheels.

#### Moving Straight

If both wheels rotate at the same speed:

Left Wheel ---> ---> --->

Right Wheel ---> ---> --->

The robot moves forward in a straight line.

### VEX Example

Suppose both drive motors are set to:

Left Motor = 50%

Right Motor = 50%

The robot drives straight toward a game object.

#### Turning Right

If the left wheel rotates faster than the right wheel:

Left Wheel ---> ---> --->

Right Wheel ---> --->

The robot turns right.

### VEX Example

Left Motor = 70%

Right Motor = 30%

The left side covers more distance than the right side, causing the robot to curve to the right.

#### Turning Left

If the right wheel rotates faster than the left wheel:

Left Wheel ---> --->

Right Wheel ---> ---> --->

The robot turns left.

#### Rotating in Place

One of the biggest advantages of differential drive robots is the ability to rotate without moving forward.

Left Wheel <---

Right Wheel --->

The robot spins about its center point.

### VEX Example

During an autonomous competition routine, a robot may need to turn exactly 90° before driving toward a goal.

The drivetrain could command:

Left Motor = -50%

Right Motor = 50%

causing the robot to rotate in place.

## Understanding Track Width (L)

To predict how a differential drive robot turns, we must know its track width (L).

Track width is the distance between the centers of the left and right wheels.

Left Wheel L Right Wheel

O ------------------ O

The larger the track width:

- The larger the turning radius

- The more space the robot needs to rotate

The smaller the track width:

- The tighter the turns

- The more agile the robot becomes

### VEX Example

Two VEX robots travel with the same wheel speeds:

- Robot A track width = 12 inches

- Robot B track width = 18 inches

Robot A will turn more sharply because its wheels are closer together.

## Real-World Example 1: Autonomous Warehouse Robots

Many warehouse robots use differential drive systems because they are simple, reliable, and highly maneuverable.

### Perception

The robot detects shelving units using:

- Cameras

- QR markers

- Distance sensors

### Planning

The navigation software determines:

- Which shelf to retrieve

- The shortest path

- Which obstacles to avoid

### Action

The robot changes wheel speeds:

Left Wheel = Faster

Right Wheel = Slower

allowing it to curve around obstacles.

### Connection to VEX

This is nearly identical to a VEX robot navigating around game elements during an autonomous routine.

Both robots:

- Use sensors for navigation

- Change wheel speeds independently

- Follow differential-drive kinematics

The primary difference is scale.

## Real-World Example 2: Robotic Vacuum Cleaners

Robotic vacuum cleaners such as those used in homes are another common example of differential drive robots.

Most robotic vacuums have:

- Two powered wheels

- One caster wheel

just like many VEX robots.

### Driving Straight

Both wheels rotate at the same speed as the vacuum cleans a room.

### Turning

When furniture is detected:

Left Motor = 40%

Right Motor = 10%

The vacuum turns around the obstacle.

### Rotating

When trapped in a corner:

Left Wheel = Reverse

Right Wheel = Forward

The vacuum spins in place until it finds a clear path.

### Connection to VEX

A VEX robot avoiding a field barrier behaves exactly the same way:

- Distance sensor detects an obstacle.

- Controller decides to turn.

- Left and right motors receive different speeds.

- Robot changes direction.

The same kinematic principles govern both systems.

## Why Differential Drive Is Popular

Differential drive robots are common because they are:

- Easy to build

- Easy to program

- Cost-effective

- Highly maneuverable

- Capable of rotating in place

Applications include:

- VEX competition robots

- Robotic vacuum cleaners

- Warehouse robots

- Service robots

- Research platforms

## Multiple-Choice Question 1

A VEX robot is driving straight toward a game object. Which wheel-speed configuration will keep the robot moving in a straight line?

A. Left Motor = 70%, Right Motor = 30%

B. Left Motor = 30%, Right Motor = 70%

C. Left Motor = 50%, Right Motor = 50% ✅

D. Left Motor = -50%, Right Motor = 50%

Correct Answer: C

Explanation:
A differential drive robot moves in a straight line when both wheels rotate at the same speed.

## Multiple-Choice Question 2

A robotic vacuum cleaner detects a chair directly in front of it and needs to turn right. Which action should its differential drive system perform?

A. Rotate both wheels at the same speed

B. Stop both wheels

C. Make the left wheel rotate faster than the right wheel ✅

D. Make the right wheel rotate faster than the left wheel

Correct Answer: C

Explanation:
When the left wheel moves faster than the right wheel, the robot follows a curved path to the right. This is the same principle used by VEX robots, warehouse robots, and robotic vacuum cleaners.