# 2.2 Forward vs. Inverse Kinematics

When working with mobile robots, engineers constantly switch between two different ways of thinking about robot motion:

[ Wheel Velocities: vL, vR ] === Forward Kinematics ===> [ Robot Motion: v, ω ]

[ Wheel Velocities: vL, vR ] <=== Inverse Kinematics === [ Desired Motion: v, ω ]

- Forward Kinematics: "If I know how fast the wheels are spinning, how will the robot move?"

- Inverse Kinematics: "If I know how I want the robot to move, what wheel speeds should I command?"

These mathematical relationships are used every day in VEX robotics, warehouse robots, robotic vacuums, autonomous vehicles, and industrial robots.

## Forward Kinematics

Let’s look at Forward Kinematics in this section. Forward kinematics calculates the robot's motion using known wheel velocities.

Suppose we know:

- Left wheel velocity: v_L

- Right wheel velocity: v_R

We can calculate:

- Linear velocity (v)

- Angular velocity (ω)

### Linear Velocity

The robot's forward speed is the average of both wheel velocities:

v=v_R+v_L2

### Angular Velocity

The robot's turning rate depends on the difference between wheel speeds:

ω=v_R-v_LL

where:

- L= track width

- ω= angular velocity

### VEX Example: Driving Straight

A VEX robot has:

Left Wheel = 1.0 m/s

Right Wheel = 1.0 m/s

Linear velocity:

v=1+12=1 m/s

Angular velocity:

ω=1-1L=0

Since there is no angular velocity, the robot travels straight ahead.

### VEX Example: Turning Right

Suppose:

Left Wheel = 1.2 m/s

Right Wheel = 0.8 m/s

Now:

v=1.2+0.82=1.0

The robot still moves forward at 1 m/s.

However:

ω≠0

because the wheel speeds are different.

The robot follows a curved path to the right.

This is how VEX robots navigate around field elements during autonomous routines.

## Real-World Example 1: Robotic Vacuum Cleaner

A robotic vacuum moves through a living room.

The left wheel rotates slightly faster than the right wheel:

Left Wheel = 0.5 m/s

Right Wheel = 0.3 m/s

Using forward kinematics, the controller predicts:

- Forward speed

- Turning rate

- Future position

As a result, the vacuum can determine whether it will avoid a chair or collide with it.

Without forward kinematics, the robot would not know where it is likely to travel next.

## Real-World Example 2: Amazon Warehouse Robot

Warehouse robots move inventory shelves through crowded fulfillment centers.

Each robot continuously reads:

- Left wheel velocity

- Right wheel velocity

Forward kinematics predicts:

- Current location

- Direction of travel

- Turning radius

The robot then updates its position on a warehouse map.

This process occurs dozens of times per second.

The exact same concept is used when a VEX robot estimates its position during autonomous navigation.

## Robot Position Updates

Once the robot's velocities are known, it can estimate how its position changes over a short time period.

### Change in Heading

Δθ=ω×Δt

### Change in X Position

Δx=v×cos⁡(θ)×Δt

### Change in Y Position

Δy=v×sin⁡(θ)×Δt

This process is called odometry.

### VEX Example: Odometry

Suppose a VEX robot:

- Starts at (0,0)

- Faces forward

- Drives for 1 second at 0.5 m/s

The robot calculates:

Distance = Speed × Time

Distance = 0.5 × 1

Distance = 0.5 m

It updates its internal position estimate accordingly.

Many advanced VEX autonomous routines use odometry for navigation.

# Multiple-Choice Question 1

A VEX robot's left and right wheels are both moving at 1.0 m/s. According to forward kinematics, how will the robot move?

A. Rotate in place

B. Move backward

C. Move forward in a straight line ✅

D. Turn left

Correct Answer: C

Explanation: When both wheel velocities are equal, angular velocity is zero, causing the robot to move straight ahead.