# 3.3 Artificial Potential Fields

While Bug algorithms make navigation decisions by following obstacle boundaries, Artificial Potential Fields (APFs) take a more continuous and intuitive approach. Instead of viewing obstacles as walls to follow, the robot imagines that it is moving through a virtual force field where goals pull it forward and obstacles push it away.

This method is inspired by concepts from physics. Just as gravity pulls objects toward a planet and magnets repel like poles, artificial potential fields create virtual forces that influence the robot's motion.

Imagine placing a ball on a sloped landscape:

- The goal is located at the bottom of a valley.

- Obstacles are located on hills.

- The ball naturally rolls toward the lowest point while avoiding the hills.

Artificial Potential Fields use this same idea to guide robotic movement.

## Attractive Fields

The robot's goal generates an attractive force.

This force continuously pulls the robot toward its target location.

Examples of goals include:

- A VEX AIM robot moving toward a scoring object

- A delivery robot moving toward a customer's front door

- A robotic vacuum returning to its charging station

As the robot moves, it continuously calculates how strongly it should be pulled toward the goal.

### VEX AIM Example

Suppose a VEX AIM robot spots a red game object using its Optical Sensor.

The robot determines:

Goal Position = (x, y)

`

An attractive force vector is generated pointing directly toward the object.

The farther away the object is, the stronger the attraction may become.

The robot continuously steers toward the object until it arrives.

## Repulsive Fields

Obstacles generate repulsive forces.

These forces push the robot away from collisions.

Common obstacles include:

- Walls

- Furniture

- Other robots

- Game field barriers

The closer the robot gets to an obstacle, the stronger the repulsive force becomes.

### VEX AIM Example

Suppose the AIM robot is approaching a game object but detects a wall nearby with its Distance Sensor.

The wall generates a repulsive force.

As the robot gets closer to the wall:

Repulsive Force ↑

``

The controller steers the robot away from the obstacle while still moving toward the target.

## Combining Forces

The robot combines all attractive and repulsive forces into a single force vector:

F*total=F*attractive+∑F_repulsive

This total force determines the robot's steering direction.

Think of the robot asking:

"Where do all the pushes and pulls suggest I should move next?"

The answer becomes the robot's heading.

## VEX AIM Example: Goal Seeking with Obstacle Avoidance

Imagine a VEX AIM robot attempting to reach a scoring goal.

### Step 1: Goal Attraction

The goal generates:

Attractive Force = Forward

### Step 2: Obstacle Detection

A field barrier appears on the robot's right side.

The barrier generates:

Repulsive Force = Left

### Step 3: Combine Forces

The robot combines both vectors:

Forward + Left

### Step 4: Action

The robot drives diagonally forward-left.

Rather than stopping and calculating a separate route, it smoothly curves around the obstacle.

This is one of the biggest advantages of Artificial Potential Fields: movement often appears very natural.

# Real-World Example 1: Robotic Vacuum Cleaners

Many robotic vacuum cleaners use behaviors that resemble potential field navigation.

### Attractive Force

The cleaning target or charging dock acts as the goal.

### Repulsive Forces

Furniture, walls, and stairs generate avoidance behavior.

### Result

The vacuum smoothly navigates around objects while continuing toward its destination.

Rather than making sharp turns at every obstacle, it gradually adjusts its path.

This creates more efficient and natural movement.

# Real-World Example 2: Autonomous Drones

Autonomous drones frequently use potential-field-inspired navigation.

### Attractive Force

The delivery location or inspection target acts as the goal.

### Repulsive Forces

The drone must avoid:

- Buildings

- Trees

- Power lines

- Other aircraft

As obstacles are detected, the navigation algorithm continuously adjusts the flight path.

The result is a smooth trajectory rather than a series of abrupt turns.

# Real-World Example 3: Warehouse Robots

Autonomous warehouse robots often navigate through crowded facilities.

Each robot is attracted toward:

- Storage shelves

- Pickup locations

- Delivery stations

However, nearby robots generate repulsive forces that encourage collision avoidance.

This allows hundreds of robots to operate in the same environment without constantly stopping.

# Advantages of Artificial Potential Fields

Potential field navigation offers several benefits:

✅ Simple mathematical framework

✅ Smooth movements

✅ Continuous obstacle avoidance

✅ Low computational cost

✅ Effective in dynamic environments

Because of these advantages, potential fields are often used in real-time robotics systems.

# The Local Minima Trap

One of the most well-known problems with Artificial Potential Fields is the local minima trap.

A local minimum occurs when the attractive and repulsive forces perfectly balance.

F_total=0

The robot experiences no net force.

As a result:

- It cannot move forward.

- It cannot move backward.

- It becomes stuck.

## VEX AIM Example: Local Minima

Suppose the AIM robot is trying to reach a goal inside a U-shaped obstacle.

Goal

X

| |

| |

| |

_____/

Robot

The goal pulls the robot forward.

However:

- The left wall pushes right.

- The right wall pushes left.

These repulsive forces cancel the attractive force.

The robot stops moving even though the goal is visible.

## Real-World Example: Delivery Robot

Imagine a sidewalk delivery robot approaching a recessed doorway.

The entrance lies inside a narrow alcove.

The goal attracts the robot into the doorway.

At the same time:

- The left wall pushes right.

- The right wall pushes left.

The robot may become trapped in front of the entrance because the forces balance.

Modern robots often combine Artificial Potential Fields with path-planning algorithms to overcome this problem.

# Multiple-Choice Question

A VEX AIM robot is moving toward a game object. The object creates an attractive force, while a nearby wall creates a repulsive force. The robot drives diagonally around the wall while continuing toward the object. Which navigation method is the robot using?

A. Bug 1 Navigation

B. Bug 2 Navigation

C. Artificial Potential Fields ✅

D. Open-Loop Control

Correct Answer: C

Explanation: Artificial Potential Fields use attractive forces that pull a robot toward goals and repulsive forces that push it away from obstacles. By combining these forces, the robot can smoothly navigate toward a target while avoiding collisions.

# Multiple-Choice Question

What is the primary cause of a local minima trap in an Artificial Potential Field?

A. The robot's battery becomes depleted.

B. The attractive and repulsive forces balance, causing the total force to equal zero. ✅

C. The robot loses communication with its sensors.

D. The wheel motors stop receiving power.

Correct Answer: B

Explanation: A local minimum occurs when the attractive force pulling the robot toward the goal is exactly balanced by repulsive forces from surrounding obstacles. The robot experiences no net force and becomes stuck before reaching the goal.