# 5.2 Dead Reckoning vs. Sensor Correction

One of the most important challenges in robotics is determining a robot's position as it moves through the world. Before a robot can navigate to a destination, avoid obstacles, or build a map, it must estimate where it is located.

Two of the most common approaches are:

- Dead Reckoning (Odometry-Based Estimation)

- Sensor Correction (Landmark-Based Localization)

Most modern robots use a combination of both methods because each has strengths and weaknesses.

## Dead Reckoning

Dead reckoning is a method of estimating position using previously known locations combined with measurements of movement.

Instead of observing the environment directly, the robot asks:

"If I started here and know how far I've moved, where should I be now?"

The robot continuously updates its position estimate using measurements from wheel encoders and the kinematic equations introduced in Chapter 2.

Starting Position

↓

Measure Wheel Rotations

↓

Calculate Distance Traveled

↓

Update Position Estimate

↓

Repeat

## Odometry

The most common form of dead reckoning in mobile robots is odometry.

Odometry uses sensors called encoders to measure wheel rotations.

For example:

1 Wheel Rotation = 0.31 meters

10 Wheel Rotations = 3.1 meters

Using wheel rotation data, the robot calculates:

- Distance traveled

- Heading changes

- Current estimated position

Using kinematics:

Δx=vcos⁡(θ)Δt
Δy=vsin⁡(θ)Δt

the robot continuously estimates its location.

## VEX AIM Example: Driving Across a Classroom

Suppose a VEX AIM robot starts at:

(0,0)

and is commanded to drive forward 2 meters.

The robot tracks wheel rotations using motor encoders.

The robot estimates:

Position = (2,0)

based entirely on wheel movement.

As long as the wheels roll perfectly, the estimate remains accurate.

## Real-World Example: Robotic Vacuum Cleaner

A robotic vacuum initially leaves its charging station.

Using wheel encoders, it estimates:

- Distance traveled

- Direction changes

- Current room position

The vacuum continuously updates its position estimate without needing to see the environment.

This allows it to begin operating immediately after startup.

However, over time errors begin to accumulate.

# Odometer Drift

The biggest weakness of dead reckoning is odometer drift.

Every measurement contains a small amount of error.

Examples include:

- Wheel slippage

- Uneven carpet

- Loose wheels

- Small bumps in the floor

- Sensor noise

Suppose a robot believes it traveled:

1.00 meters

but actually traveled:

0.98 meters

That 2 cm error may seem insignificant.

However:

After 10 movements = 20 cm error

After 50 movements = 1 meter error

The robot's position estimate gradually drifts away from reality.

## VEX AIM Example: Wheel Slip

Imagine a VEX AIM robot driving across a smooth classroom floor.

The wheels briefly slip during a turn.

The encoders report:

Wheel Rotations = 100

but the robot actually moved only 95 rotations worth of distance.

The AIM robot now believes it is somewhere it never actually reached.

As additional movements occur, the error compounds.

## Real-World Example: Warehouse Robots

Warehouse robots travel thousands of meters every day.

If they relied only on dead reckoning:

Small Encoder Error

+

Thousands of Movements

=

Large Position Error

Eventually the robot may:

- Miss a shelf

- Stop in the wrong location

- Collide with infrastructure

This is why modern warehouse robots continuously correct their position estimates.

# Sensor Correction

To prevent drift, robots periodically compare their estimates against real observations of the environment.

This process is called sensor correction.

Instead of asking:

"Where do my encoders think I am?"

the robot asks:

"What do my sensors actually see?"

By comparing expected observations to actual observations, the robot can correct accumulated error.

## Landmarks

Sensor correction often relies on recognizable features called landmarks.

A landmark is a feature that:

- Does not move

- Is easy to recognize

- Has a known location

Examples include:

- Wall corners

- Field barriers

- Doors

- AprilTags

- Fixed posts

- Charging stations

The robot stores these known locations in a landmark map.

## VEX AIM Example: Using a Wall Corner

Suppose a VEX AIM robot believes its coordinates are:

(5.2, 3.1)

The robot then detects a classroom corner using its Distance Sensor.

The known landmark is stored at:

(5.0, 3.0)

The observation reveals that the robot's estimate is slightly incorrect.

The localization system updates its position estimate:

Old Estimate: (5.2, 3.1)

Corrected Estimate: (5.0, 3.0)

The accumulated drift is reduced.

## VEX AIM Example: Optical Landmark Detection

Imagine an AIM robot using an Optical Sensor to identify a brightly colored marker on a classroom wall.

The marker acts as a landmark.

Whenever the robot sees the marker:

- Measure distance to the marker.

- Measure angle to the marker.

- Compare to expected values.

- Update position estimate.

This is similar to how many professional robots perform localization.

## Real-World Example: Self-Driving Cars

Self-driving vehicles continuously use landmarks for localization.

Common landmarks include:

- Lane markings

- Traffic signs

- Curbs

- Utility poles

- Buildings

The vehicle compares camera and LiDAR observations with stored map data.

When discrepancies are found, the vehicle adjusts its position estimate.

This significantly reduces odometry error.

## Real-World Example: Mars Rovers

Mars rovers experience significant wheel slip when driving over sand and rocks.

If the rover used only odometry:

Estimated Position ≠ Actual Position

The rover instead uses:

- Cameras

- Terrain features

- Rock formations

to identify landmarks.

These observations help correct accumulated drift and improve navigation accuracy.

# Why Modern Robots Use Both

Dead reckoning provides:

✅ Fast updates

✅ Simple calculations

✅ Works immediately

But suffers from:

❌ Accumulating error

❌ Drift

❌ Wheel slip problems

Sensor correction provides:

✅ High accuracy

✅ Drift reduction

✅ Real-world verification

But may require:

❌ Detectable landmarks

❌ Additional sensors

❌ More computation

For this reason most autonomous robots use:

Dead Reckoning

+

Sensor Correction

=

Reliable Localization

This combination forms the foundation of modern SLAM systems.

# Multiple-Choice Question

A VEX AIM robot estimates its position using wheel encoder measurements. After several minutes of operation, the robot's estimated position no longer matches its actual location. What is the most likely cause?

A. Path planning

B. Odometer drift ✅

C. Occupancy grids

D. Artificial potential fields

Correct Answer: B

Explanation:
Dead reckoning relies on continuously adding movement estimates. Small errors from wheel slippage, uneven surfaces, or sensor noise accumulate over time, causing odometer drift.

# Multiple-Choice Question

Which of the following is an example of sensor correction?

A. Using wheel rotations to estimate distance traveled.

B. Averaging motor speeds during a turn.

C. Using a known wall corner to update a robot's position estimate. ✅

D. Increasing motor power when climbing a ramp.

Correct Answer: C

Explanation:
Sensor correction occurs when a robot observes a known landmark and uses that observation to adjust its internal position estimate, reducing accumulated drift.

# CodeLens Activity

Consider the following pseudocode:

distance_traveled = encoder_rotations * wheel_circumference

robot_x = robot_x + distance_traveled

### Question

What localization method is being used?

A. Landmark Localization

B. Sensor Correction

C. Dead Reckoning ✅

D. SLAM Mapping

Correct Answer: C

Explanation: The robot is estimating its position using encoder measurements and previously estimated positions. This is the definition of dead reckoning.

# CodeLens Activity: Landmark Correction

Consider the following pseudocode:

estimated_x = 5.4

estimated_y = 2.8

if landmark_detected:

estimated_x = 5.0

estimated_y = 3.0

### Question

What is happening when the landmark is detected?

A. The robot is increasing motor speed.

B. The robot is correcting accumulated odometry error. ✅

C. The robot is generating a new occupancy grid.

D. The robot is running A* pathfinding.

Correct Answer: B

Explanation: The robot compares its estimated location with information obtained from a known landmark. The resulting correction reduces accumulated drift and improves localization accuracy.